### 1. Overall Objective

The robot is designed to autonomously navigate its environment without human intervention. It detects obstacles using an ultrasonic sensor and controls four DC motors independently using an Arduino Uno and motor drivers. The logic ensures the car moves forward, stops at obstacles, reverses, and resumes its path, implementing basic autonomous navigation.

### 2. Sensor Logic

Ultrasonic Sensor (HC-SR04)

- The sensor continuously measures the distance to objects in front of the car.

- Working:

  - Arduino triggers a short ultrasonic pulse (TRIG pin).

  - The sensor emits the pulse, which bounces off obstacles.

  - The echo is received by the sensor (ECHO pin).

  - Arduino calculates the distance using the formula:

Distance (cm)= Time of flight×0.0342)/2

- Logic:

  - If distance > 2 cm → continue moving forward.

  - If distance ≤ 2 cm → stop, reverse, and resume after a delay.

### 3. Motor Control Logic

Four-Motor System

- Each wheel has an independent motor connected to an H-bridge motor driver (L293D).

- H-bridge allows bidirectional control (forward/reverse).

- Motors are controlled via the Arduino using the following logic:

  - Forward Movement: All motors receive positive voltage → wheels rotate forward.

  - Reverse Movement: All motors polarity reversed → wheels rotate backward.

  - Stop: Motor driver disables output → wheels freewheel.

Speed Control

- PWM signals from Arduino regulate motor speed.

- Logic example:

  - analogWrite(speedPin, 150) sets motor speed to moderate value for controlled movement.

### 4. Main Algorithm Flow

- Initialization

  - Arduino initializes serial communication.

  - Motor drivers and ultrasonic sensor pins are configured.

  - Motor speeds set to default forward movement.

- Loop Execution

  - Continuously measure distance using ultrasonic sensor.

  - Decision Branch:

    - If obstacle detected (≤ 2 cm):

      - Stop all motors.

      - Reverse for a fixed duration.

      - Stop and then resume forward motion.

    - Else:

      - Keep moving forward.

  - This loop repeats, allowing autonomous navigation.

### 5. Four-Motor Traction Logic

Why Four Motors?

- Independent wheel control improves traction and handling.

- Voltage is equally divided among motors to prevent overloading and ensure balanced movement.

- Logic ensures:

  - Equal forward movement → straight path.

  - Equal reverse movement → straight reverse.

- Any adjustment for turning can be added by slowing one or two wheels independently.

### 6. Energy Conversion Logic

- Batteries provide electrical energy (3.7V, 1800mAh each).

- Motors convert electrical energy into mechanical energy.

- Some energy is lost as heat → real-world efficiency < 100%.

- Logic ensures that motors receive a voltage sufficient to rotate wheels and overcome friction and drag.

### 7. Obstacle Avoidance Logic

- The car continuously checks the front distance.

- Obstacle handling:

  - Detect: Distance ≤ 2 cm triggers logic.

  - Stop: Immediately stop motors.

  - Reverse: Move backward for a fixed duration to create space.

  - Resume Forward: Return to normal forward motion.

- Optional extensions:

  - Detect left/right deviation using additional sensors.

  - Turn instead of simple reverse.

### 8. Arduino Implementation Logic

Code Structure

- Include Libraries: For motor and sensor control.

- Define Pins: For motors, ultrasonic sensor.

- Setup Function:

  - Initialize serial monitor for debugging.

  - Initialize motors and sensors.

- Loop Function:

  - Read sensor distance.

  - Compare with threshold.

  - Apply motor control functions (forward, reverse, stop) based on conditions.

- Motor Functions:

  - forwardMotors() → all motors forward.

  - reverseMotors() → all motors backward.

  - stopMotors() → disable all motors.

### 9. Summary of Decision Logic

| Condition | Action |
| --- | --- |
| Distance > 2 cm | Move Forward |
| Distance ≤ 2 cm | Stop → Reverse → Forward |
| Continuous loop | Autonomous navigation |

- Logic is simple, reactive, and relies solely on real-time sensor data.

- Can be extended for advanced behaviors (turning, path planning).

### ✅ Key Points

- Sensors detect environment → Arduino processes data → Motors respond accordingly.

- Four-motor system allows smooth movement and independent traction.

- Algorithm continuously cycles, creating autonomous behavior.

- Obstacle avoidance ensures collision-free navigation.

- Energy management ensures proper battery distribution to all motors.